# ATLAS Master Flightline Clock - System Review & Analysis

Document: System Architecture & Gap Analysis
Project: ATLAS Master Flightline Clock
Date: July 3, 2026
Purpose: Foundation document for ICD-001 generation and feature implementation
Analyst: Abacus AI Agent

## 1. Executive Summary

Project ATLAS is a functional prototype for a Master Flightline Clock designed for competitive aviation events. The system has evolved from basic JSON configuration to a professional, database-driven appliance architecture running on a Mac Mini M4. The codebase demonstrates solid architectural separation between timing logic, persistence, UI, and display rendering.

### Current State: v1.5.0 - Mission-Ready Prototype

Key Strengths: - Clean MVC architecture with modular design - Hardware-agnostic timing engine - Adaptive 6-row LED display strategy (128x64 and 64x64 profiles) - SQLite persistence with audit trail foundation - Professional operator console with custom navigation

Critical Gaps: - No physical hardware interface (simulator only) - Missing Interface Control Document (ICD-001) for LED hardware - Auto-boot appliance functionality not implemented - Hardware abstraction layer (HAL) not present

## 2. System Architecture Analysis

### 2.1 Current Architecture Overview

The system implements a clean Model-View-Controller (MVC) pattern with the following layers:

┌─────────────────────────────────────────────────────────────┐
│                        main.py                              │
│              (Application Wiring & Entry Point)             │
└─────────────────────────────────────────────────────────────┘
                            │
        ┌───────────────────┼───────────────────┐
        ▼                   ▼                   ▼
┌───────────────┐   ┌──────────────┐   ┌──────────────────┐
│  Core Engine  │   │     GUI      │   │   Persistence    │
│               │   │              │   │                  │
│  engine.py    │◄─►│ windows.py   │◄─►│  database.py     │
│  models.py    │   │ themes.py    │   │  (SQLite)        │
│  timers.py    │   │ led_sim.py   │   │                  │
└───────────────┘   └──────────────┘   └──────────────────┘
        │                   │
        └─────────┬─────────┘
                  ▼
        ┌──────────────────┐
        │  Display Output  │
        │                  │
        │ AtlasLEDSimulator│
        │  (6-row render)  │
        └──────────────────┘

### 2.2 Component Breakdown

#### Core Engine (engine.py)

- Responsibility: State machine and timing logic

- Update Rate: 1 Hz (1000ms ticker)

- State Model: Phases P0-P4 based on UTC time comparison

- Signals: model_updated, brightness_changed

- Independence: Fully decoupled from UI and display hardware

Key Features: - Automatic heat activation (15 minutes before track open) - Phase calculation based on track_open, track_close, heat_end timestamps - Countdown formatting (MM:SS) - Heat cancellation logic

State Transition Logic:

Phase 0 (ARMED):       now < (track_open - 300s)  → "TRACK OPENS IN"
Phase 1 (PRE-ENTRY):   now > (track_open - 300s)  → [Same as P0]
Phase 2 (TRACK ENTRY): track_open to track_close  → "ENTER NOW" (Green)
Phase 3 (ACTIVE):      track_close to heat_end    → "HEAT TIME REMAINING" (Red)
Phase 4 (COOLDOWN):    now > heat_end             → "HEAT COMPLETED"

#### Persistence Layer (database.py)

- Technology: SQLite3 embedded database

- Location: data/atlas.db

- Tables:

  - heats: heat_number, thermalling_dir, track_open, track_close, heat_end, status

  - settings: key-value storage (competition_name, display_width, display_height, led_scale, brightness)

CRUD Operations: - save_heat(), get_todays_schedule(), get_active_heat(), get_next_pending_heat() - set_heat_status(), cancel_active_heat(), delete_heat() - save_setting(), get_setting()

Audit Trail Foundation: Heat status transitions (READY → ACTIVE → COMPLETED/CANCELLED) are preserved, not deleted.

#### Data Models (models.py)

- HeatConfig: Pydantic model for heat scheduling data

- AtlasDisplayModel: Display state container broadcasted by engine

AtlasDisplayModel:
  - local_time: str          # "HH:MM:SS"
  - heat_number: str         # "01", "02", etc.
  - thermalling_dir: str     # "LEFT" or "RIGHT"
  - planned_open: str        # "HH:MM"
  - planned_close: str       # "HH:MM"
  - primary_timer: str       # "MM:SS"
  - primary_timer_label: str # "TRACK OPENS IN", "ENTER NOW", etc.
  - primary_timer_color: str # Hex color "#FFFF00"
  - band_color: str          # Status bar color
  - is_armed: bool           # System active state

#### Operator Console (windows.py)

- Framework: PySide6 (Qt6)

- Style: Fusion (cross-platform consistency)

- Navigation: Custom button bar (QStackedWidget) - replaced native QTabWidget due to macOS styling limitations

Three Tabs:

- LIVE Tab:

  - Real-time heat status display

  - “ABORT / CANCEL ACTIVE HEAT” button

  - Updates via engine.model_updated signal

- SCHEDULE Tab:

  - Heat list table (QTableWidget)

  - Add/Edit/Delete heat functionality

  - Form inputs: heat_number, thermalling_dir, track_open, track_close, heat_end

  - Row selection for editing

  - Validation: track_close must be after track_open

- SETTINGS Tab: ✅ ALREADY IMPLEMENTED

  - Competition name input

  - Display size dropdown: “128x64 (Large)” / “64x64 (Compact/Half)”

  - LED scale input (simulator pixel size)

  - Brightness slider: 10-100%, labeled (Night/Overcast/Normal/Bright Day/Full Sun)

  - Live brightness updates (no restart required)

  - “SAVE & APPLY” button

  - System Controls Section:

    - ✅ “RESTART ATLAS” button (os.execv process replacement)

    - ✅ “EXIT TO MACOS” button (QApplication.quit)

Graceful Shutdown: closeEvent() includes placeholder comment for hardware clear frame:

# HARDWARE - Send CLEAR/OFFLINE frame to LED panels here before exit

#### LED Simulator (led_simulator.py)

- Purpose: Public display simulation (represents physical LED matrix)

- Rendering: Custom bitmap font (5x7), circular LED pixels

- Adaptive Layout: Automatically selects Full or Compact mode based on display width

6-Row Display Strategy:

Full Mode (128x64):

Row 1: Local Time              (Medium, Cyan)
Row 2: Heat Info (H01 LEFT)    (Medium, Yellow)
Row 3: Windows (O 14:30 C 14:35) (Small, Gray)
Row 4: Phase Label              (Small, Phase Color)
Row 5: Primary Timer            (Large 2x, Phase Color)
Row 6: Status Band              (2px height, Phase Color)

Compact Mode (64x64):

Row 1: Local Time              (Centered, Cyan)
Row 2: Heat + Direction        (Centered, Yellow)
Row 3: Track Open Time         (Centered, Gray)
Row 4: Track Close Time        (Centered, Gray)
Row 5: Phase Label (abbreviated) (Centered, Phase Color)
Row 6: Primary Timer            (Centered, Phase Color)

Label Abbreviations (Compact Mode): - “TRACK OPENS IN” → “OPEN IN” - “ENTER NOW” → “ENTER” - “HEAT TIME REMAINING” → “HEAT LEFT” - “WAITING” → “WAIT”

Brightness Control: - Live adjustment via slider (10-100%) - _apply_brightness() method scales RGB values - Immediate repaint without restart - Emits via engine.brightness_changed signal

#### Supporting Modules

themes.py: - COCKPIT_BG: Dark background (#0A0A0A) - BTN_MASTER_ARM: Blue action buttons (#0066CC) - BTN_CANCEL: Red abort buttons (#880000) - TABS: Custom tab styling (unused after migration to button navigation)

timers.py: - AtlasTimerEngine: 10Hz ticker with signal-based countdown - Currently unused in main application (engine.py uses its own 1Hz QTimer) - Kept for potential future precision timing needs

config.py: - AtlasConfig: Legacy JSON configuration loader - Currently unused (replaced by database settings) - Calculates display dimensions from panel counts (P10 panel math: 32px wide, 16px high) - Kept for reference/future hardware panel configuration

## 3. Documentation Coverage Analysis

### 3.1 Existing Documentation

| Document | Status | Completeness | Quality |
| --- | --- | --- | --- |
| atlas_system_handover_dossier_2026-07-03.pdf | ✅ Complete | High | Mission-ready handover summary |
| atlas_technical_update_2026-07-02.docx | ✅ Complete | High | Development progress narrative |
| atlas_sds_001_software_design_spec.pdf | ✅ Complete | Medium-High | 3 pages, covers timing/persistence/rendering |
| ATLAS-DEP-001_Deployment_Guide.pdf | 🟡 Draft | Medium | Outlines appliance philosophy, needs implementation details |
| tm_001_traceability_matrix.pdf | 🟡 Draft | Low-Medium | Basic requirement mapping, needs test procedures |
| ATLAS-ICD-001 (Interface Control Document) | ❌ Missing | 0% | CRITICAL GAP - No LED hardware interface spec |

### 3.2 Documentation Quality Assessment

Strengths: - Professional tone and structure - Clear architectural principles - GAMP 5 influence (audit trail, traceability mindset) - Tonal alignment with user preferences (casual, technical, concise)

Weaknesses: - ICD-001 completely absent (hardware interface undefined) - TM-001 has no test procedures or verification methods - ATLAS-DEP-001 lacks specific launcher scripts and installation steps - No user manual or operator quick-reference guide - No hardware commissioning procedures

## 4. Gap Analysis: Requested Features vs. Current State

### 4.1 Feature Request Assessment

| Feature | Status | Implementation Notes |
| --- | --- | --- |
| Settings Tab with Display Type Dropdown | ✅ IMPLEMENTED | Line 172-178 in windows.py: QComboBox with “128x64 (Large)” and “64x64 (Compact/Half)” options. Persists to database. |
| Auto-boot to Clock | ❌ MISSING | Mentioned in ATLAS-DEP-001 and TM-001 (URS-009) as “Planned”. No launcher script exists. |
| “Exit to macOS” Button | ✅ IMPLEMENTED | Line 234-238 in windows.py: “EXIT TO MACOS” button with QMessageBox confirmation, calls QApplication.quit(). |
| Architectural Separation (Engine vs Display) | ✅ GOOD FOUNDATION | Engine emits display model via signals. LED simulator is decoupled. Hardware abstraction layer missing. |

### 4.2 Critical Observation: Display Type Ambiguity

The user request mentions “LED Module” vs “LCD Monitor” as display types, but the current implementation has: - “128x64 (Large)” → Implies LED matrix or high-resolution output - “64x64 (Compact/Half)” → Implies smaller LED matrix

Potential Interpretation Gap: 1. Option A: User wants conceptual display output type selector: - “LED Module” (physical hardware) → then choose resolution (64x64, 128x64, 256x32, etc.) - “LCD Monitor” (simulated on computer screen) → fullscreen/windowed mode

- Option B: User wants the existing dropdown renamed to clarify intent:

  - “128x64 LED Matrix (Full Layout)”

  - “64x64 LED Matrix (Compact Layout)”

- Option C: User wants both:

  - Display Type: LED Module / LCD Monitor (top-level selector)

  - LED Resolution: 64x64 / 128x64 / 256x32 (conditional, only if LED Module selected)

Recommendation: Clarify with user whether existing implementation satisfies requirement or if architectural change is needed.

## 5. Technical Specifications for ICD-001

### 5.1 LED Hardware Context

Based on code comments and configuration patterns, the system is designed for P5/P10 LED matrix panels:

P10 Panel Specifications (Standard Outdoor Module): - Resolution: 32×16 pixels per module - Pixel Pitch: 10mm - Module Dimensions: 320mm × 160mm - Scan Rate: 1/8 or 1/16 scan - Color Depth: RGB (16-bit typical) - Brightness: 5000-8000 cd/m² (outdoor rated)

P5 Panel Specifications (Higher Density): - Resolution: 64×32 pixels per module - Pixel Pitch: 5mm - Module Dimensions: 320mm × 160mm - Scan Rate: 1/16 or 1/32 scan

Current Configuration Targets: - 64×64 Display: 2×4 P10 modules (2 wide, 4 high) = 640mm × 640mm - 128×64 Display: 4×4 P10 modules (4 wide, 4 high) = 1280mm × 640mm

### 5.2 ICD-001 Required Sections

To generate a comprehensive Interface Control Document, the following information is needed:

#### 5.2.1 Physical Interface Specifications

Required Information: - [ ] Connector types (HUB75, HUB08, or proprietary) - [ ] Power requirements (voltage, current per panel, total power budget) - [ ] Cable specifications (length limits, shielding requirements) - [ ] Environmental ratings (IP rating, temperature range, UV resistance)

HUB75 Interface (Most Common):

Pin Assignment:
R1, G1, B1   - RGB data for upper half
R2, G2, B2   - RGB data for lower half
A, B, C, D   - Row address lines
CLK          - Clock signal
LAT          - Latch signal
OE           - Output enable (brightness control via PWM)
GND          - Ground

Electrical Characteristics (Typical): - Logic Level: 3.3V or 5V TTL - Clock Frequency: 1-20 MHz - Current Draw: 2-8A per 32×16 module at full brightness

#### 5.2.2 Communication Protocols

Three Primary Options:

Option 1: Direct GPIO Control (Raspberry Pi) - Bit-banged HUB75 protocol - Libraries: rpi-rgb-led-matrix (C++), rgbmatrix (Python wrapper) - Pros: Lowest latency, no external controller - Cons: Requires Raspberry Pi, CPU-intensive, limited panel count

Option 2: Serial/SPI Controller - Dedicated LED controller board (e.g., Novastar, Linsn, Colorlight) - Protocol: Proprietary serial or Ethernet-based - Pros: Offloaded processing, supports large arrays - Cons: Requires protocol documentation, more expensive

Option 3: Network-Based (Artnet/E1.31) - DMX-over-Ethernet protocols - Controllers: PixLite, ESPixelStick, WLED - Pros: Flexible, long cable runs, standard protocols - Cons: Network latency, overkill for static displays

Current Codebase Assumption: None specified. Simulator is software-only.

#### 5.2.3 Data Format & Timing

Frame Structure (HUB75 Direct Drive):

For 64×64 display (4096 pixels):
- Frame Rate Target: 60-120 Hz (16.7-8.3ms per frame)
- Bit Depth: 8-bit per color (24-bit RGB)
- Data Size: 64 × 64 × 3 bytes = 12,288 bytes per frame
- Scan Pattern: Row-by-row interlaced (upper/lower half simultaneously)

Timing Requirements: - Refresh Rate: Minimum 60 Hz to avoid flicker - Update Latency: <100ms for countdown updates (1Hz update is acceptable) - Brightness PWM: 100-4000 Hz (perceptual linearity via gamma correction)

#### 5.2.4 Coordinate System & Mapping

Atlas Display Model to Physical Pixels:

Atlas Logical Coordinates:
  Origin: Top-left (0, 0)
  X-axis: Left to right (0 to display_width-1)
  Y-axis: Top to bottom (0 to display_height-1)

Panel Chaining (for 128×64 = 4×4 P10 modules):
  [Panel 0,0] [Panel 1,0] [Panel 2,0] [Panel 3,0]
  [Panel 0,1] [Panel 1,1] [Panel 2,1] [Panel 3,1]
  [Panel 0,2] [Panel 1,2] [Panel 2,2] [Panel 3,2]
  [Panel 0,3] [Panel 1,3] [Panel 2,3] [Panel 3,3]

Data Stream Mapping: Serpentine or linear (controller-dependent)

Color Space: - Input: Hex RGB (“#00FFFF” → R=0, G=255, B=255) - Output: May require gamma correction (typical gamma=2.2 for perceptual linearity)

#### 5.2.5 Hardware Abstraction Layer Design

Proposed Architecture:

# Abstract Base Class
class DisplayBackend(ABC):
    @abstractmethod
    def initialize(self, width, height) -> bool:
        """Initialize hardware/connection"""
        pass
    
    @abstractmethod
    def render_frame(self, model: AtlasDisplayModel):
        """Render the current display model"""
        pass
    
    @abstractmethod
    def set_brightness(self, percent: int):
        """Set brightness 0-100%"""
        pass
    
    @abstractmethod
    def clear(self):
        """Blank the display"""
        pass
    
    @abstractmethod
    def shutdown(self):
        """Clean hardware shutdown"""
        pass

# Concrete Implementations
class LEDSimulatorBackend(DisplayBackend):
    """Current implementation - software simulation"""
    pass

class HUB75DirectBackend(DisplayBackend):
    """Direct GPIO control for Raspberry Pi"""
    pass

class NetworkBackend(DisplayBackend):
    """Artnet/E1.31 network protocol"""
    pass

class SerialControllerBackend(DisplayBackend):
    """Serial/USB LED controller protocol"""
    pass

Backend Selection:

# In main.py or config
backend_type = db.get_setting("display_backend", "simulator")

if backend_type == "simulator":
    display = LEDSimulatorBackend(config)
elif backend_type == "hub75":
    display = HUB75DirectBackend(config)
elif backend_type == "network":
    display = NetworkBackend(config)
# ... etc

### 5.3 ICD-001 Open Questions

To finalize the Interface Control Document, the following must be determined:

- Primary Hardware Target:

  - ☐ Raspberry Pi 4/5 with direct HUB75 control?

  - ☐ Dedicated LED controller (which brand/model)?

  - ☐ Network-based DMX controller?

  - ☐ Custom PCB/FPGA solution?

- Panel Specifications:

  - ☐ P5 or P10 modules?

  - ☐ Indoor or outdoor rated?

  - ☐ Specific supplier/model number?

  - ☐ Power supply specifications?

- Control Computer:

  - ☐ Mac Mini M4 (current) - requires USB/Ethernet adapter

  - ☐ Raspberry Pi 4/5 - native GPIO

  - ☐ Windows PC - USB/serial controller

  - ☐ Dual system (Mac for operator, Pi for display)?

- Cable Run:

  - ☐ Distance from computer to LED panels?

  - ☐ Indoor/outdoor cabling requirements?

  - ☐ Power cable separation from data cables?

- Failsafe Behavior:

  - ☐ Display state on application crash?

  - ☐ Display state on power loss/restore?

  - ☐ Watchdog requirements?

## 6. Code Structure Analysis

### 6.1 Directory Structure

Project_Atlas/
├── src/
│   ├── main.py                 (45 lines)  - Entry point, application wiring
│   ├── core/
│   │   ├── models.py           (24 lines)  - Pydantic data models
│   │   ├── database.py         (88 lines)  - SQLite CRUD operations
│   │   └── engine.py           (91 lines)  - State machine & timing logic
│   └── gui/
│       ├── windows.py          (303 lines) - Operator console (3 tabs)
│       ├── themes.py           (49 lines)  - Stylesheet definitions
│       ├── led_simulator.py    (179 lines) - Public display renderer
│       ├── fonts.py            (not provided, referenced in led_simulator.py)
│       └── timers.py           (41 lines)  - Precision timer (currently unused)
├── data/
│   └── atlas.db                (SQLite database - not in upload)
├── docs/
│   ├── atlas_sds_001_software_design_spec.pdf
│   ├── atlas_technical_update_2026-07-02.docx
│   ├── atlas_system_handover_dossier_2026-07-03.pdf
│   ├── ATLAS-DEP-001_Deployment_Guide.pdf
│   └── tm_001_traceability_matrix.pdf
└── (Other files not provided: run_atlas.sh, fonts.py)

Total Python Code: ~820 lines (excluding fonts.py and launcher scripts)

### 6.2 Code Quality Assessment

Strengths: - ✅ Clean separation of concerns (MVC respected) - ✅ Pydantic models for type safety - ✅ Signal-based decoupling (Qt signals/slots) - ✅ Database abstraction with SQLite row_factory for dict results - ✅ Adaptive rendering based on display dimensions - ✅ Graceful shutdown patterns - ✅ Professional error handling (QMessageBox confirmations)

Weaknesses: - ⚠️ timers.py implemented but unused (engine.py uses QTimer directly) - ⚠️ config.py legacy JSON loader still present but unused - ⚠️ No unit tests or integration tests - ⚠️ No logging framework (only print statements) - ⚠️ fonts.py not provided (missing dependency for led_simulator.py) - ⚠️ No hardware interface abstraction (simulator is tightly coupled)

Technical Debt: - Remove or refactor timers.py (redundant with engine.py QTimer) - Remove or repurpose config.py (superseded by database settings) - Add proper logging (Python logging module) - Create abstract display backend interface - Add validation tests for phase transition logic

### 6.3 Dependencies

Identified from Imports: - PySide6 (Qt6 for Python) - pydantic - sqlite3 (standard library) - time, os, sys (standard library)

Likely Additional Dependencies (not visible in provided code): - fonts.py with AtlasBitmapFont class (custom module) - Potentially Pillow/PIL for future image rendering

## 7. Feature Implementation Recommendations

### 7.1 Settings Tab Enhancement (if needed)

Current State: ✅ Already has display dropdown, brightness slider, restart/exit buttons

If “LED Module vs LCD Monitor” distinction is required:

Option 1: Rename Existing Dropdown

self.combo_disp_size.addItems([
    "LED Matrix 128x64 (Full Layout)",
    "LED Matrix 64x64 (Compact Layout)",
    "LCD Monitor Fullscreen"  # <-- New option
])

Option 2: Two-Level Selection

# Display Type Selector
self.combo_display_type = QComboBox()
self.combo_display_type.addItems(["LED Module", "LCD Monitor"])
self.combo_display_type.currentIndexChanged.connect(self._on_display_type_changed)

# Resolution Selector (conditional visibility)
self.combo_resolution = QComboBox()
self.combo_resolution.addItems(["64x64", "128x64", "256x32"])
self.combo_resolution.setVisible(display_type == "LED Module")

Backend Selection:

# In settings tab
self.combo_backend = QComboBox()
self.combo_backend.addItems([
    "Simulator (Development)",
    "HUB75 Direct (Raspberry Pi)",
    "Network (Artnet/E1.31)",
    "Serial Controller"
])

### 7.2 Auto-Boot Implementation

macOS LaunchAgent Implementation:

Step 1: Create LaunchAgent plist

<!-- ~/Library/LaunchAgents/com.atlas.flightline.plist -->
<?xml version="1.0" encoding="UTF-8"?>
<!DOCTYPE plist PUBLIC "-//Apple//DTD PLIST 1.0//EN" 
  "http://www.apple.com/DTDs/PropertyList-1.0.dtd">
<plist version="1.0">
<dict>
    <key>Label</key>
    <string>com.atlas.flightline</string>
    
    <key>ProgramArguments</key>
    <array>
        <string>/usr/bin/python3</string>
        <string>/Applications/Atlas/main.py</string>
    </array>
    
    <key>RunAtLoad</key>
    <true/>
    
    <key>KeepAlive</key>
    <dict>
        <key>SuccessfulExit</key>
        <false/>
    </dict>
    
    <key>StandardOutPath</key>
    <string>/tmp/atlas.log</string>
    
    <key>StandardErrorPath</key>
    <string>/tmp/atlas.error.log</string>
</dict>
</plist>

Step 2: Enable Automatic Login

# macOS System Preferences
sudo defaults write /Library/Preferences/com.apple.loginwindow autoLoginUser "atlas_operator"

Step 3: Disable Sleep/Screensaver

sudo pmset -a displaysleep 0
sudo pmset -a sleep 0
defaults -currentHost write com.apple.screensaver idleTime 0

Step 4: Installation Script

#!/bin/bash
# install_atlas.sh

echo "Installing ATLAS Master Flightline Clock..."

# Copy application files
sudo mkdir -p /Applications/Atlas
sudo cp -r src/* /Applications/Atlas/

# Install LaunchAgent
cp com.atlas.flightline.plist ~/Library/LaunchAgents/
launchctl load ~/Library/LaunchAgents/com.atlas.flightline.plist

# Configure system
sudo pmset -a displaysleep 0
defaults -currentHost write com.apple.screensaver idleTime 0

echo "Installation complete. Reboot to test auto-start."

### 7.3 Hardware Abstraction Layer Implementation

Step 1: Create Abstract Interface

# display_backend.py
from abc import ABC, abstractmethod
from core.models import AtlasDisplayModel

class DisplayBackend(ABC):
    """Abstract base class for display output backends."""
    
    @abstractmethod
    def initialize(self, config) -> bool:
        """Initialize display hardware/connection."""
        pass
    
    @abstractmethod
    def render(self, model: AtlasDisplayModel):
        """Render the current display state."""
        pass
    
    @abstractmethod
    def set_brightness(self, percent: int):
        """Set display brightness 0-100%."""
        pass
    
    @abstractmethod
    def clear(self):
        """Clear/blank the display."""
        pass
    
    @abstractmethod
    def shutdown(self):
        """Graceful shutdown and cleanup."""
        pass

Step 2: Refactor Existing Simulator

# led_simulator_backend.py
from display_backend import DisplayBackend
from gui.led_simulator import AtlasLEDSimulator

class SimulatorBackend(DisplayBackend):
    def __init__(self):
        self.window = None
    
    def initialize(self, config) -> bool:
        self.window = AtlasLEDSimulator(config)
        self.window.show()
        return True
    
    def render(self, model: AtlasDisplayModel):
        if self.window:
            self.window.update_model(model)
    
    def set_brightness(self, percent: int):
        if self.window:
            self.window.set_brightness(percent)
    
    def clear(self):
        # Render empty/black frame
        pass
    
    def shutdown(self):
        if self.window:
            self.window.close()

Step 3: Create Hardware Backend Stub

# led_hardware_backend.py
from display_backend import DisplayBackend

class HUB75Backend(DisplayBackend):
    def __init__(self):
        self.matrix = None
    
    def initialize(self, config) -> bool:
        try:
            from rgbmatrix import RGBMatrix, RGBMatrixOptions
            options = RGBMatrixOptions()
            options.rows = 16
            options.cols = 32
            options.chain_length = 4  # For 128x64
            options.parallel = 2
            self.matrix = RGBMatrix(options=options)
            return True
        except ImportError:
            print("ERROR: rgbmatrix library not available")
            return False
    
    def render(self, model: AtlasDisplayModel):
        if not self.matrix:
            return
        
        # TODO: Render model to matrix
        # self.matrix.Clear()
        # Draw text, shapes to self.matrix
        # Use graphics library to convert AtlasDisplayModel to pixels
        pass
    
    def set_brightness(self, percent: int):
        if self.matrix:
            self.matrix.brightness = int(percent * 100)
    
    def clear(self):
        if self.matrix:
            self.matrix.Clear()
    
    def shutdown(self):
        if self.matrix:
            self.matrix.Clear()
            # HUB75 hardware doesn't need explicit shutdown

Step 4: Backend Factory

# backend_factory.py
from display_backend import DisplayBackend
from led_simulator_backend import SimulatorBackend
from led_hardware_backend import HUB75Backend

def create_display_backend(backend_type: str) -> DisplayBackend:
    """Factory function to instantiate display backends."""
    
    if backend_type == "simulator":
        return SimulatorBackend()
    elif backend_type == "hub75":
        return HUB75Backend()
    elif backend_type == "network":
        # return NetworkBackend()
        raise NotImplementedError("Network backend not yet implemented")
    else:
        raise ValueError(f"Unknown backend type: {backend_type}")

Step 5: Update main.py

# main.py (modified)
from backend_factory import create_display_backend

def main():
    app = QApplication(sys.argv)
    app.setStyle(QStyleFactory.create("Fusion"))
    
    db = AtlasDatabase()
    engine = AtlasEngine(db)
    config = DisplayConfig(db)
    
    # Backend selection from settings
    backend_type = db.get_setting("display_backend", "simulator")
    display_backend = create_display_backend(backend_type)
    
    if not display_backend.initialize(config):
        print("ERROR: Failed to initialize display backend")
        sys.exit(1)
    
    # Connect engine to backend
    engine.model_updated.connect(display_backend.render)
    engine.brightness_changed.connect(display_backend.set_brightness)
    
    # Launch operator console
    operator_win = OperatorWindow(engine)
    operator_win.show()
    
    sys.exit(app.exec())

### 7.4 Exit to macOS Button Enhancement

Current Implementation: ✅ Already exists (line 234-238 in windows.py)

Potential Enhancement: Confirmation with Heat Status

def _shutdown_app(self):
    active_heat = self.engine.db.get_active_heat()
    
    if active_heat:
        warning_msg = f"WARNING: Heat {active_heat['heat_number']} is currently ACTIVE!\n\n"
        warning_msg += "Shutting down will stop the public display.\n"
        warning_msg += "Are you sure you want to exit?"
        
        reply = QMessageBox.warning(self, 'Active Heat Warning', warning_msg,
                    QMessageBox.Yes | QMessageBox.No, QMessageBox.No)
    else:
        reply = QMessageBox.question(self, 'System Shutdown', 
                    "Are you sure you want to exit Atlas?",
                    QMessageBox.Yes | QMessageBox.No)
    
    if reply == QMessageBox.Yes:
        # Hardware cleanup before exit
        # TODO: Call display_backend.shutdown()
        from PySide6.QtWidgets import QApplication
        QApplication.quit()

## 8. ICD-001 Generation Strategy

### 8.1 Document Structure Proposal

ATLAS-ICD-001: LED Display Interface Control Document

Sections: 1. Introduction - Purpose and Scope - Applicable Documents - Terminology and Abbreviations

- System Overview

  - Architecture Diagram (Mac Mini → Controller → LED Panels)

  - Data Flow (AtlasEngine → DisplayBackend → Physical Hardware)

  - Interface Boundaries

- Physical Interface Specifications

  - Connector Pinouts (HUB75/HUB08)

  - Cable Requirements

  - Power Specifications

  - Mechanical Mounting

- Electrical Characteristics

  - Voltage Levels

  - Current Requirements

  - Signal Timing Diagrams

  - EMC/EMI Considerations

- Communication Protocol

  - Data Frame Structure

  - Timing Requirements

  - Handshaking (if applicable)

  - Error Detection/Correction

- Data Mapping

  - Coordinate System

  - Color Space (RGB bit depth, gamma)

  - Brightness Control (PWM frequency, linear/log mapping)

  - Panel Addressing Scheme

- Software Interface

  - DisplayBackend Abstract Class Specification

  - Backend Implementation Requirements

  - State Machine Integration Points

  - Error Handling Requirements

- Performance Requirements

  - Refresh Rate (minimum 60 Hz)

  - Update Latency (maximum 100ms)

  - Brightness Range (10-100%)

  - Environmental Operating Conditions

- Testing & Validation

  - Interface Compliance Tests

  - Performance Benchmarks

  - Acceptance Criteria

- Appendices

  - Example Code Snippets

  - Vendor Datasheets

  - Troubleshooting Guide

### 8.2 Information Gathering Needed

To complete ICD-001, the following must be provided by stakeholders:

Hardware Selection: - [ ] LED panel manufacturer/model - [ ] Controller board type - [ ] Computer interface (GPIO, USB, Ethernet)

Installation Environment: - [ ] Indoor or outdoor installation - [ ] Viewing distance (affects pixel pitch selection) - [ ] Ambient light conditions (affects brightness requirements) - [ ] Power availability (voltage, current capacity)

Performance Requirements: - [ ] Acceptable refresh rate (Hz) - [ ] Maximum update latency (ms) - [ ] Brightness adjustment resolution (10% steps vs. 1% steps)

Regulatory Requirements: - [ ] Electrical safety standards (UL, CE, SAA) - [ ] EMC compliance requirements - [ ] Environmental ratings (IP rating)

### 8.3 Provisional ICD-001 Approach

Given Current Information Gaps:

Strategy: Generate a modular ICD-001 with: - Section A: Generic LED matrix interface theory (applicable to any hardware) - Section B: Example implementations (HUB75, Network, Serial) - Section C: Placeholder sections for specific hardware (to be completed after procurement)

Benefit: Provides architectural guidance immediately while allowing hardware-specific details to be added later.

## 9. Implementation Priority & Roadmap

### Phase 1: Documentation (Immediate)

- ✅ Generate ATLAS-ICD-001 (modular/generic version)

- ✅ Document Hardware Abstraction Layer design

- Update TM-001 with new requirements (URS-011: HAL, URS-012: Backend selection)

### Phase 2: Code Refactoring (Week 1)

- Create DisplayBackend abstract class

- Refactor LEDSimulator into SimulatorBackend

- Add backend selection to Settings tab

- Update main.py to use backend factory

- Add logging framework (replace print statements)

### Phase 3: Auto-Boot Implementation (Week 1-2)

- Create LaunchAgent plist for macOS

- Write installation script (install_atlas.sh)

- Test automatic startup on reboot

- Document uninstall procedure

### Phase 4: Hardware Integration (Pending Procurement)

- Acquire LED panels and controller

- Implement HUB75Backend or SerialControllerBackend

- Test frame rendering and brightness control

- Finalize ICD-001 with actual hardware specifications

- Commissioning and field testing

### Phase 5: Production Hardening (Post-Hardware)

- Add unit tests (pytest for engine logic)

- Add integration tests (hardware interface validation)

- Implement watchdog process

- Create backup/restore functionality

- Write operator training manual

## 10. Open Questions & Decisions Required

### 10.1 User/Stakeholder Decisions

- Display Type Terminology:

  - Is existing “128x64 (Large)” / “64x64 (Compact)” dropdown sufficient?

  - Or is a “LED Module” vs “LCD Monitor” conceptual distinction needed?

  - Should there be a backend selector (Simulator, Hardware, Network)?

- Hardware Platform:

  - Will physical LED panels use Raspberry Pi + HUB75?

  - Or dedicated LED controller (Novastar, Linsn, etc.)?

  - Or network-based DMX/Artnet controller?

- Panel Specifications:

  - P5 (5mm pitch, higher density) or P10 (10mm pitch, standard)?

  - Indoor or outdoor rated?

  - Specific supplier/model number?

- ICD-001 Detail Level:

  - Generate generic/architectural version now, finalize later?

  - Or wait until hardware is procured for complete specifications?

- Auto-Boot Priority:

  - Implement macOS LaunchAgent now (can test with simulator)?

  - Or defer until hardware is ready?

### 10.2 Technical Decisions

- Rendering Library:

  - Continue with custom bitmap font renderer?

  - Or integrate Pillow/PIL for more flexible text/graphics?

  - Or use Qt’s QPainter to render to pixel buffer, then send to hardware?

- Brightness Mapping:

  - Linear brightness (0-100% → PWM 0-100%)?

  - Or gamma-corrected (perceptual linearity)?

  - Or exponential curve (better low-light perception)?

- Error Handling:

  - If hardware connection fails, fall back to simulator?

  - Or display error and require restart?

  - Or show hardware error on operator console?

- Multi-Display Support:

  - Should operator console and public display be on separate computers?

  - Or continue single-computer architecture?

- Configuration Migration:

  - Keep backward compatibility with config.py?

  - Or enforce database-only configuration?

  - Or provide import/export for config portability?

## 11. Conclusion & Next Steps

### 11.1 Summary

The ATLAS Master Flightline Clock is a well-architected, functional prototype with solid foundations: - ✅ Clean MVC separation - ✅ Database-driven persistence - ✅ Adaptive 6-row display rendering - ✅ Professional operator console - ✅ Settings tab with display selection, brightness control, and system controls already implemented

Critical gaps identified: - ❌ No Interface Control Document (ICD-001) for LED hardware - ❌ No Hardware Abstraction Layer (only simulator exists) - ❌ Auto-boot launcher not implemented - ❌ No physical hardware integration

### 11.2 Immediate Deliverables

This analysis enables: 1. Generation of ATLAS-ICD-001 (modular/architectural version, hardware details TBD) 2. Implementation of HAL (DisplayBackend abstract class + refactored simulator) 3. Auto-boot launcher script for macOS 4. Settings tab enhancements (if “LED Module vs LCD Monitor” distinction is truly needed)

### 11.3 Recommended Actions

For User/Product Owner: 1. Clarify display type dropdown requirements (current implementation vs. desired behavior) 2. Confirm hardware platform for LED panels (Raspberry Pi, controller, network) 3. Approve ICD-001 modular approach (architectural now, hardware specs later)

For Development: 1. Generate ATLAS-ICD-001 document (architectural/generic version) 2. Implement DisplayBackend abstract class and refactor simulator 3. Create macOS LaunchAgent for auto-boot 4. Add backend selector to Settings tab (Simulator, HUB75, Network, Serial)

For Hardware Procurement: 1. Specify LED panel requirements (P5/P10, indoor/outdoor, dimensions) 2. Select controller/driver approach (direct GPIO, serial, network) 3. Finalize ICD-001 with actual hardware specifications post-procurement

Document Status: Complete
Next Document: ATLAS-ICD-001 (Interface Control Document)
Author: Abacus AI Agent
Review Date: July 3, 2026